PLASTERING : The process of covering rough surfaces with a plastic material  to obtain an even,smooth,regular clean and durable surface is called Plastering.

PURPOSE: Plastering conceals defective workmanship and covers up unsound and cheap quality material. Besides plastering is required to provide a satisfactory   base for decorating the surface by white-washing, colour washing , distempering or painting.

INGREDIENTS:

1.Cement         		 2. lime or clay     		 3. Fine aggregate

4. Water   			 5.  Plaster of paris		 6.   Keene’s cement

7. Barium Plaster  		 8.  Acoustical plaster etc.

TYPES OF PLASTERING:

- Lime plaster

- Cement plaster

- Mud plastering

- Stucco plastering

- Plaster on lath

PLASTERING PROCESS :

- Plaster is applied with a trowel in three or more coats according to traditional practice. The first coat (called the render, scratch coat or coarse stuff) is designed to fill voids and even the surface.

- The next coat is of similar but finer material is referred to as the levelling or float, as it is finished with a wooden float ready to accept a finish of fine stuff.

- Finishing coats of lime and plaster of Paris (called setting stuff or finestuff) are applied with a steel trowel and can be worked up to a verysmooth and glassy finish.

- Coarse stuff was traditionally reinforced with animal hair or chopped grass. Its proportions are three parts aggregate (sand) to one part of binder (lime and reinforcing material, often with cement added). The first coat is roughly trowelled on and its surface is scored to provide a key to accept the next coat.

- The floating coat may contain a higher proportion of sand aggregate than the render and a small proportion of fine reinforcing material such as animal hair. It is ruled off with a long, true levelling batten to set up a perfectly regular finish to receive the finishing coat.

DEFECTS  IN PLASTER WORK:    The following defects may occur in plaster work :

- Cracking

- Blowing or blistering of plaster

- Falling out of plaster

- Efflorescence