Public Transport Technologies

Key technological characteristics:

(a)	Support - contact between vehicle and surface

•	rubber tire on concrete

•	steel wheel on steel rail

•	others

(b)	Guidance - lateral control:

•	steered by driver

•	guided by track

•	others

(c)	Propulsion:

•	diesel ICE:  conventional or clean

•	LNG, CNG

•	electric motor

•	Hybrid, others

(d)	Control

•	manual/visual

•	manual/signal

•	automatic:  ATO, ATC

Guidance  and Control Technology

- - Lateral kerbs and side rollers (O-BAHN)

- - An optical guidance (steering linked to a video-monitoring system and a road marking recognition system : CIVIS of Irisbus and Matra Transport International,

- - A single centrally embedded rail (of tram type) with disconnectable steering bogies (GLT) or arms supporting tilting rollers in V form (TRANSLOHR),

- - An inductive cable guidance system (electronic steering by means of inductive cables embedded in the roadway) (Cegelec-AEG system),

- -A magnetic guidance system under design (magnetic steering servitude to an electric cable embedded in the roadway) which has been envisaged for STREAM.

- - The TVR and the TRANSLOHR can both run on a single wire light electric supply of the tramways type as the return current travels via the guiding rail,

- - The CIVIS uses electric wheel motor, which means an integral low floor between the wheels,

- - The STREAM power system consists of an electric contact line embedded in a roadway which has been adapted for urban purposes as voltage is only achieved beneath the vehicle.

Bus:  vehicles operating individually with rubber tires, with manual lateral and longitudinal control

Key decisions:

Vehicles size:  minibus (20 passengers)

up to bi-articulated (165+ passengers)

Vehicle design:  high floor or low floor

Right-of-way:  all options are available

Guidance:  is guided operation appropriate

at some locations?

Propulsion:  all options available

Fare payment:  on-vehicle or off-vehicle

Light Rail: vehicles operating individually or in short trains with electric motors and overhead power collector, steel wheel on steel rail with manual or automatic longitudinal control

Key decisions:

Vehicle design: high floor or low floor,

articulated or rigid body

Right-of-way:  all options available

Operating arrangements:

automated or manually driven

Heavy Rail/Metro: vehicles operating in trains with electric motors on fully separated rights-of-way with manual signal or automatic longitudinal control; level boarding, off-vehicle fare payment

Key decisions:

Train length:  affects station cost

Right-of-way:  at-grade, elevated, or tunnel

Station spacing

Operating arrangements:  degree of automation

Commuter Rail:  vehicles operating in trains with long station spacing, serving long trips into central city, large imbalance between peak hour and other period ridership.

Example: KTM

Traditional Transit Services

•	Bus on shared right-of-way

•	Streetcar on shared right-of-way

•	Heavy rail on exclusive right-of-way

•	Commuter/Regional rail on semi-exclusive right-of-way

New/Recent Service Concepts

•	Bus Rapid Transit

•	Light Rail on exclusive right-of-way

Increasing Diversity

•	Driver arrangements:  part-timers, 10-hour days, pay by vehicle type

•	Routing and scheduling:  fixed, flexible, advance booking

•	Vehicle types:  minibuses, articulated buses and railcars, bi-level railcars, low-floor

•	Control options:  fixed block, moving block, manual, ATO, ATC

•	Priority options:  full grade separation, semi-exclusive right-of-way, signal pre-emption

•	Dual mode operations:  bus, light rail

Rail advantages:

•	High capacity

•	Lower operating costs

•	Better service quality

•	Stronger land use influence

•	Fewer negative externalitites

Bus advantages:

•	Low capital costs

•	Wide network coverage

•	Single vehicle trips

•	Flexibility

•	“Dual mode” nature

Urban transport system

- Factors influencing urban transport pattern

  - Infrastructure investment choices: expressway/LRT/Metro/Busway

  - Income changes and economic development (Income and car ownership)

  - Interaction of transport and urban form and the influence of urban planning policy

  - Prices and economic instruments (car/fuel price/tax; KL vs. Singapore)

  - Technology choice (NMT/MC/car/rail vs. expressway)

  - Transport patterns and infrastructure influence the urban form that evolves in any particular city and, conversely, the urban form of a city influences its transport patterns and further infrastructure investments.

City and transport

- Walking City

  - High density

  - Mixed use

  - Short distance origin and destination patterns highly dispersed throughout the city

- Transit city

  - High density city centre with mixed use

  - Short distance origin and destination patterns within city centre

  - Long distance and radial origin-destination pattern with satellite/suburb township

- Automobile city

  - High density commercial use core

  - Commercial, retail and industrial landuse separated and dispersed throughout the city

  - Long distance origin-destination pattern highly dispersed throughout the city