Water Demand Calculation

• Water requirement can be calculated

1. Residential water demand.

2. Industrial water demand.

3. Water demand for commercial services.

4. Water demand for fire fighting

Reference

- Occupancy types are set according to BNBC 2006 (table8.5.1(a))

- Water demand is set according to BNBC 2006 (table8.5.1(d))

Peak factors:

Residential zone = 2.4

Industrial zone = 1.4

Administrative zone = 1.4

Common Facility zone = 4

Fire Fighting = 1

Water Demand for Industrial Zone

Assumption:

- Per day production is assumed as 10000lb/day

- Water requirement 1.8 L/lb (source:http://www.yarnsandfibers.com/preferredsupplier/reports_fullstory.php?id=475&section=&p_type=General&country=Global)

Water Requirement for Fire Fighting

Assumptions for Calculating Fire Hydrant Water Demand:

1. Each point must be served by at least two fire hydrants at a time

2. Nozzle diameter of the hydrant is 3 inches (2r)

3. Velocity of water in the pipe is 3 fps (v)

4. Each hydrant will supply water for 30 minutes (t)