# Chapter 2

# Design of Slab bridge

## 2.1 Design of rail post

### SQ –

- Difference between effective and clear span

- Draw all the working diagrams

- S20H16 mean?

he simple difference is an “H” series truck is a two-axle truck, while an “HS” series truck has multiple loaded axles.

- Effective depth for rail = D-2.5 and Effective depth for post = D-1.5

### Problems –

- Load calculation for railing, W

Self weight(6 x 612 x 12 x 150) + vertical given load

- Moment calculation

W.Actual Spacing of railing8

- Spacing of rail post 6.5’. Clear span of length of slab bridge 14.5’. Determine actual spacing

Number of rail post = 14.56.5+1 = 4

Sa = 14.54-1=4.83 '

- Effective depth of railing 3.5’ or d=6’ and vertical moment, Mv = 546.77 lb-ft. Calculate required steel and do a shear check

- Maximum Horizontal load at base at rail post

Horizontal load x C/C x height of 1st rail + Horizontal load x C/C x height of 2nd rail

## 2.2 Design of edge beam

### SQ

- What is edge beam?

- Edge beam is that part of the bridge deck which is situated along the side of the deck slab.

- It is used for the distribution and transmission of the traffic loads close to edges of the bridge deck slab.

- It accommodates the railings and lighting poles and also directs the rain water to flow through the drainage system, protecting the underside of bridge deck

- What is kerb?

Kern indicates boundary between pavement and shoulder, sometimes as island, footpath or kerb parking space

- Deff =Edge beam height – 2.375

- What are the loads and moment that will carried by edge beam?

- Self-weight of edge beam + rail post wight

- D.L. moment + L.L. Moment

- Other name of that edge beam?

Kerb

## 2.3 Design of Slab

- H-S loading details

- What do you mean by impact analysis and design shear, moments

## 2.4 Design of Abutment

- Forces acting on abutment?

Lateral earth pressure, surcharge, wheel load